BeeWare Project Reports – Henry-Léo Martin

16/09/2026

- System Operating Principles & Operational Concept

The objective of this engineering project is the development of an autonomous drone capable of detecting foraging Asian hornets (Vespa velutina), executing close-proximity tracking during their flight back to their colony, and localizing the concealed nest in complex or cluttered environments (e.g., dense forest canopies, urban structures).

  - Multi-Phase Operational Architecture

Phase 0 — Ground Watch & Priming: The UAV remains stationed on a ground pad equipped with a protein/sugar-based attractant. The primary compute unit monitors the bait zone continuously with low idle power draw.

Phase 1 — In-Situ Detection & Edge Classification: Insects visiting the bait are segmented and classified directly on an edge hardware accelerator. The system must discriminate between Vespa velutina, Apis mellifera (honeybees), Vespa crabro (European hornets), and common social wasps with zero cloud reliance.

Phase 2 — Autonomous Takeoff & Dynamic Pursuit: Upon detecting an outbound flight vector, the UAV arms, transitions to flight mode, and executes a visual/acoustic pursuit at a safe standoff distance (1.5 to 3 m). Yaw and pitch tracking rely on a monochrome global shutter camera to eliminate high-frequency motion blur caused by wing beat cycles (~100–150 Hz), augmented by a 4-microphone array computing Time Difference of Arrival (TDoA) beamforming.

Phase 3 — Terminal Nest Localization: Once tracking ceases or the target approaches high foliage, the drone switches to wide-area thermal infrared (TIR) mapping to detect the metabolic thermal signature of the colony (typically 30–35 °C against ambient background), validated by acoustic signature analysis.

Phase 4 — Telemetry & Reporting: The UAV locks target GNSS coordinates, logs high-resolution multispectral imagery, and transmits an actionable mission intelligence packet to the base operator before initiating Return-To-Launch (RTL).

  - System Hardware & Software Stack Overview

| Subsystem | Hardware / Specification | Functional Role |
| --- | --- | --- |
| Compute Core | Raspberry Pi 5 | Orchestration, navigation state machines, trajectory generation. |
| AI Accelerator | 13 TOPS dedicated Neural Processing Unit | Real-time object detection and insect classification at high FPS. |
| Flight Controller | Pixhawk 2.4.8 | Low-level rate/attitude control, mixer, failsafe management. |
| Propulsion | 2700 kV brushless motors, 4-in-1 70A ESC | Dynamic thrust response for rapid acceleration during interception. |
| Primary Vision | Monochrome Global Shutter Sensor | High-frame-rate target centroid tracking without rolling shutter skew. |
| Acoustic Array | 4-channel microphone geometry + ADC/DSP | TDoA spatialization for bearing and elevation angle estimation. |
| Thermal Vision | Long-Wave Infrared (LWIR) core | Heat-differential mapping of active hornet nests. |
| Middleware | ROS 2 (Robot Operating System 2) | Inter-process communication (DDS), node modularity, determinism. |
| Bridge Protocol | UART (/dev/ttyAMA0, 921600 bps) | Low-latency state sync between Pixhawk and ROS 2 companion node. |

- Work Completed

  - Structural Fabrication & Post-Processing (PETG Airframe)

Main Frame Post-Processing:

- I collected the central body of the drone from the 3D printer.

- I removed all the support structures from the print.

Launching New Prints:

- Front head: Designed to hold the global shutter camera and front microphones.

- Rear part: Designed to hold the LiPo battery and balance the center of gravity of the drone.

  - Research on ROS 2 on Raspberry Pi 5

I studied how to set up the software architecture on the Raspberry Pi 5 using ROS 2 (Robot Operating System 2):

Operating System Choice: Ubuntu Server 64-bit (24.04 LTS or 22.04 LTS) is the standard choice to run ROS 2 reliably on ARM64 boards, rather than standard Raspberry Pi OS.

ROS 2 Distribution: Evaluated ROS 2 Jazzy (latest LTS) and ROS 2 Humble (very stable with many compatible packages).

Communication with Pixhawk: Looked into connecting the Raspberry Pi 5 to the Pixhawk via a serial UART link (/dev/ttyAMA0 at 921600 baud) using either:

- Micro-XRCE-DDS (if using PX4 autopilot firmware) for direct ROS 2 topic exchange.

- MAVROS (if using ArduPilot firmware) over the MAVLink protocol.