BeeWare Project Reports – Henry-Léo Martin

21/09/2026

- Session Objectives

- Disassemble the previous drone testbed and transfer all components into the new 3D-printed frame.

- Mount the Raspberry Pi 5 alongside the existing avionics.

- Switch the flight controller operating system from ArduPilot to PX4 to simplify external companion computer integration.

- Perform baseline sensor and radio calibrations under the new firmware.

- Work Completed & Applied Methodology

2.1. Mechanical Rebuild & Insert Installation

- Disassembled the entire previous drone build to recover the components.

- Installed heat-set threaded inserts into the new frame to properly seat and screw down each component (Pixhawk, ESC, motors, and Raspberry Pi 5).

- Mounted all internal modules and the Raspberry Pi 5 into their dedicated spots in the new frame.

  - OS Migration: ArduPilot to PX4

- Erased ArduPilot and flashed PX4 on the flight controller.

- Reason: Connecting an external companion computer (Raspberry Pi 5) for autonomous drone control is more straightforward and native with PX4 (direct DDS integration with ROS 2).

  - Initial Calibrations

Started the routine sensor and radio calibrations in QGroundControl:

- Accelerometer: Completed the multi-orientation calibration steps.

- GPS / Compass: Connected and initialized.

- Remote Control (RC): Calibrated stick ranges, channels, and switches.

- Results

- All hardware components and the Raspberry Pi 5 are securely mounted on the new frame using threaded inserts.

- PX4 firmware is successfully installed and recognized.

- Baseline calibration sequence is underway (accelerometer, GPS, and radio verified)

- Next Steps

- Complete any remaining sensor calibrations (such as outdoor compass calibration).

- Wire and test the communication link between the Pixhawk (PX4) and the Raspberry Pi 5.

- Verify motor rotation directions and arming sequences under PX4.